Raw material handling

PTA Silo / PTA Feed Hopper / PTA Feeder

- PTA moisture content and particle size (affects dissolution rate in slurry)

- Conveying N₂ purity / oxygen content (prevents PTA discoloration during pneumatic transfer)

- Silo level, feeder speed, feed rate to slurry tank (kg/hr)

EG Storage (MEG)

- MEG purity: water content and diethylene glycol (DEG) content, both affect reaction stoichiometry and final polymer quality

- Storage temperature, tank level

Slurry Mixing Tank (D-1200)

- PTA:MEG molar ratio (typically ~1:1.2–1.5, sets stoichiometry for esterification)

- Slurry temperature and agitation speed (homogeneity, prevents settling)

- Catalyst/additive dosing rate

- Tank level, slurry pump discharge pressure and flow

Esterification

1st ES Reactor (R-2100)

- Reaction temperature (typically ~250–260°C)

- Reactor pressure (slightly above atmospheric, allows water vapor to leave)

- Residence time (per the diagram, ~2.4 hr based on the 30-ton size)

- Acid value (COOH) and ES% (esterification percent), sampled every 8 hr per the notes

- Level in each of the three internal chambers, agitation speed

1st ES Column

- Column top temperature (must separate water vapor from MEG vapor cleanly, MEG should reflux back, water should leave, this prevents the reverse hydrolysis reaction the diagram calls out)

- Reflux ratio, water removal rate, pressure drop

2nd ES Reactor (R-2200)

- Temperature per chamber/zone (5 chambers, horizontal design for added residence time)

- Toner (color additive) dosing at chamber 1, stabilizer dosing at chamber 5

- Acid value target (~400 meq/kg) and ES% target (~96%), sampled every 4 hr per the notes

- Degree of polymerization of the oligomer outlet (target ~5 monomers connected)

- Level per chamber

2nd ES Column

- Same critical parameters as the 1st ES column: top temperature, MEG reflux, water removal rate

Monomer Pump / Pre-PC Filter

- Oligomer flow rate and viscosity (temperature-dependent)

- Filter differential pressure (ΔP), fouling/cleaning frequency

Polycondensation

1st PC Reactor (R-3100)

- Melt temperature (typically ~270–280°C)

- Vacuum level (moderate vacuum at this stage, e.g. tens of mbar)

- Residence time, agitator torque (early indicator of viscosity rise)

- MEG removal rate via the direct condenser

1st/2nd/3rd PC Direct Condensers (using EG)

- Condenser temperature and EG circulation rate

- MEG vapor condensation efficiency

- Recycled EG purity (it's reused as the condensing medium)

Ejectors (vacuum system)

- Motive EG/steam pressure and flow

- Achieved vacuum level and its stability (vacuum creep here directly affects downstream IV control)

2nd PC Reactor (R-3200)

- Melt temperature (higher than 1st PC stage)

- Deeper vacuum than 1st PC stage

- Residence time, agitator torque (viscosity tracking)

- MEG removal rate

3rd PC Reactor / Finisher (R-3300)

- Final melt temperature (typically ~280–290°C)

- Deepest vacuum of the whole train

- Residence time

- Final Intrinsic Viscosity (IV), this is the single most critical quality output of the entire polymerization train

- Agitator torque (commonly used as a real-time proxy/control feedback for IV, since direct IV measurement is slow)

- Melt color (b-value) and acetaldehyde (AA) content

- Level (per the diagram's note, inlet/outlet are at the bottom since the reactor runs under vacuum)

Chip making and storage

Polymer Pump / Pre-cutter Filter

- Discharge pressure, melt temperature uniformity

- Filter ΔP (removes gels/contamination before cutting)

Chip Cutters (parallel units)

- Strand quench water temperature

- Cutter blade speed/sharpness

- Chip size uniformity, cut consistency

Chip Receiver

- Conveying air flow/pressure (per the diagram, air-driven)

- Chip moisture pickup, chip integrity (breakage/dust)

Chip Silo / Off-Grade Silo

- Chip moisture content (PET is strongly hygroscopic, this is critical for downstream quality)

- Silo temperature (prevents sticking/agglomeration)

- Off-grade classification criteria (IV out of spec, color, contamination)

Solid State Polycondensation (SSP)

- SSP temperature (typically ~200–220°C, below melt point)

- Residence time (many hours, often 10–20+ hr)

- N₂ purge flow and purity (removes byproduct MEG/water, prevents oxidative discoloration)

- Chip moisture before entering SSP (must be very low to avoid hydrolysis reversing the IV gain)

- Final IV target after SSP, AA reduction

Polymer melt distribution (5-way valve → 3-way valve → spin beams)

- TIRS103 / PIRS103 — polymer temperature and pressure right after the five-way valve (inlet from PM2, SF1, SF2, SF3, draw off), this is your incoming Hi-IV melt monitoring point

- TIRS104 / PIRS104 — temperature/pressure downstream, before the three-way valve

- TIRS105 / PIRS105 and TIRS106 / PIRS106 — temperature/pressure on each branch after the three-way valve splits to SF1A and SF1B headers

- TIRS109A/B, PIRS108A/B and TIRS110A/B, PIRS110A/B — temperature and pressure at the individual beam headers feeding TW-1A and TW-1B

- Five-way and three-way valve position — these are sequence/state-controlled by the DCS (which inlet is open) rather than a continuous PID loop, but valve position feedback is a real-time monitored point

- Polymer Hi-IV viscosity (shown next to TIRS103/PIRS103, range 0.635–0.870) — this one's worth flagging as borderline: if your plant has an inline melt viscometer at this point (which the tag-style presentation suggests), it's a genuine continuous DCS signal. If it doesn't, this number is more likely a reference range tied to the lab IV result from the polycondensation finisher, not a live reading. Worth confirming with your instrumentation list.

Quench air system (per spin position)

- Temperature °C (TIC)

- Pressure, mm H2O (PIC)

- Velocity/airflow, m/sec (FIC)

- Humidity, %RH (AIC/HIC, if an online RH probe is fitted)

Finish oil application

- Concentration % — controlled via the oil/water metering ratio (FIC ratio control); genuinely real-time if there's an inline refractometer or conductivity sensor providing feedback, otherwise it's set as a fixed metering ratio and verified periodically by lab OPU%

Drawing / draw rolls

- Feed roll speed (SIC)

- Draw roll speed (SIC) — labeled "Sunflower" on your diagram

- Draw ratio — calculated continuously from the feed roll/draw roll speed ratio, and is itself often the actual control setpoint operators adjust

Cantravering / tow collection

- Doff time — PLC-logged event timestamp, not a control loop but a real-time logged parameter

- No. of position — configuration/count value

- Batch No. — assigned tracking ID, used to stamp the time-series data for traceability rather than something that's "controlled"

What stays off the DCS (lab/QC, from the "Spun Tow Quality" box)

Spun denier, tenacity, elongation, abnormal yarn count, H2O%, and OPU% are all physical tests run on tow samples in the lab. None of these run continuously on PLC/DCS unless the line has online denier or OPU sensors installed, which isn't indicated on this diagram. Same pattern as acid value/ES%/IV on the polymerization side: these are the quality results you'd eventually want a soft sensor predicting from the temperature, pressure, draw ratio, and quench air signals above, rather than something you can pull straight off the historian.

Here's the PFD for the drawline section, going from the spun-tow cans you just collected through to baled staple fiber.

V

visualize



V

visualize show_widget



Now the critical parameters, following the same real-time vs. lab split as before.

Tow creel / can feed

- Tow band tension (TIC) — via dancer roll or load cell

- Creel/feed roll speed (SIC) — line speed reference point

- Can changeover/empty-can detection — sensor-triggered, drives splice or alarm logic

Drawing (hot draw bath)

- Draw bath water temperature (TIC) — critical, typically 70–95°C; this single loop has the biggest real-time effect on tenacity and elongation

- Draw bath level (LIC)

- Steam pressure to bath heater (PIC), if steam-heated

- Feed roll speed and draw roll speed (SIC ×2) — across the multiple stands

- Draw ratio — calculated continuously from the feed:draw roll speed ratio; this is the actual setpoint operators tune

- Tow tension before/after the draw stand (TIC)

- Overall line speed (SIC) — master reference that everything downstream syncs to

Finish oil application

- Applicator roll speed (SIC)

- Finish concentration — controlled via oil:water metering ratio (FIC ratio control); genuinely real-time only if there's an inline refractometer or conductivity probe, otherwise it's a fixed metering setpoint verified later by lab OPU%

- Bath level (LIC)

- Applicator roll temperature (TIC), if heated

Stuffer box crimper

- Crimper box back-pressure (PIC) — set by the pressure plate, directly controls crimp intensity

- Feed roll overfeed speed (SIC) relative to line speed — sets crimp frequency

- Crimper box steam pressure/temperature (PIC/TIC), if steam-heated to thermoset the crimp

- Crimp frequency — calculated from the overfeed ratio, a derived real-time value even though crimp count verification is often still a lab/visual check

Relaxation / heat-setting oven

- Oven temperature, per zone (TIC ×multiple)

- Conveyor/belt speed (SIC) — sets residence time

- Hot air circulation flow (FIC)

- Residence time — calculated from belt speed and oven length

Cutter

- Cutter blade speed (SIC)

- Feed roll speed into cutter (SIC)

- Cut length — set via the feed roll : blade speed ratio, a configured/calculated parameter

- Motor load/torque (AI) — secondary signal for blade wear or jam detection

Baling

- Bale press force/pressure (PIC)

- Bale weight (WIC) — load-cell based, typically the trigger that ends a press cycle

- Press stroke/position — sequence-controlled, not a continuous PID loop

- Bale density — calculated from weight and compressed volume